BALBOA WORKING

PRIMING MODE SCREEN:

- Power On, Priming Mode for some time and then switches to the normal working mode.

- Can operate Jet1, Jet2 and Circulation, independent of each other. Circulation is not synced with Jet1 or Jet2.

- Manually exit the Priming mode by pressing the back-arrow button in upper left corner.

NORMAL WORKING SCREEN:

- Top Left Corer

  - “SLEEP BUTTON” icon

  - “TEMPERATURE RANGE” icon

  - “HEAT MODE” icon

- Centre of the screen

  - “TEMPERATURE” – Current and Set temp.

Sleep Icon:

- Single touch - Turn off display

- Touch display again - Turn on display

Temp Range Icon:

- L – Low

- H – High

Heating Mode Icon:

- R – Ready

- Resting R – Rest

- RR – Ready/Rest

Temperature:

- “CURRENT TEMPERATURE” in Black colour.

- Up Arrow – Increase Set Temp by 0.5 Degree

- Down Arrow – Decrease Set Temp by 0.5 degree

- “SET TEMPERATURE” will be shown in Yellow Colour, while setting the temp, for a brief period of time

Temperature Range:

- LOW

  - Set temp from 10 to 37 degrees

- HIGH

  - Set temp from  26.5 to 40 degrees

Heat Mode:

- “READY”. (Heating States: “NOT HEATING”, “HEATING’ & “HEAT WAITING”)

  - Measured Temp < Set Temp by 1 Degree or more

    - HEATING

      - Red “Thermometer with Increasing Temperature” icon

    - HEAT WAITING – for 1-2 Min

      - Red “Blinking Thermometer” icon

    - Heating State again; Until current temp reaches to Set temp

  - Measured Temperature > Set Temperature by 1 degree or more

    - STOP HEATING

      - No Thermometer icon.

(“CIRCULATION PUMP” starts occasionally. Cannot be operated manually while in a Normal Working).

- “REST”

  - By switching into it, Heating stops immediately

  - Jet1, Jet2, & Blower (if were running) stop after some time

- “READY/REST or READY-IN-REST”

  - Operating Jet1 in the Rest Mode, switches the mode to the Ready/Rest Mode.

  - Spa-Controller works normally as in Ready Mode.

  - Does not switch back to the Rest Mode automatically.